# XR BUILD (v4.1)

Technical Brief

____________________________________________________________________________________

Last edited by: Albert on 16 Feb 2026

Project: Side Quest: The Leaning Tower of Regolith (ARC 2026)

Target Sub-Team: XR team

____________________________________________________________________________________

# 1. Executive Summary

The system provides a low-latency, stereoscopic "Robot’s Eye View" coupled with a dedicated Command Highway for robotic actuation. We utilize an Asymmetric Hybrid Architecture: high-speed video is handled via raw TCP sockets for maximum VR fluidity, while robotic logic and kinematics utilize the ROS 2 (Jazzy/Humble) framework. This allows an operator in a Meta Quest 3 to switch viewpoints and authorize robotic actions instantly via a physicalized VR control console.

# 2. Hardware Stack

- Sensors: 2x Luxonis OAK-D Pro PoE (Standalone units).

  - Camera A (.222): "The Inspector" (High-Oblique/Top-Down).

  - Camera B (.223): "The Site Manager" (Side-Elevation).

- Mediator: Raspberry Pi 5 (8GB) with Active Cooling.

- Actuator: Dobot Magician E6 (6-DOF Robotic Arm). Static IP: 192.168.5.1.

- Networking: 1Gbps Wired Backbone (PoE Switch + HP USB-C Dock G5).

  - Pi Interface (eth0): Multi-subnet configuration. Primary: 169.254.1.10 (Cameras); Alias: 192.168.5.10 (Robot).

- Operator Station: Windows 11 Laptop running Unity 6 (URP).

- HMD: Meta Quest 3 (via Quest Link).

# 3. System Architecture & Data Flow

## 3.1. The Robot Site (Hardware-Accelerated PoE)

To maximize the Pi 5's CPU availability for future Computer Vision tasks, the video pipeline has been offloaded to the OAK-D’s internal Myriad X VPU.

- Hardware Encoding: Left and Right mono sensors are compressed into JPEGs directly on the camera chip using the VideoEncoder node.

- Temporal Sync: A Sync node ensures that Left/Right frame pairs are captured at the exact same millisecond, eliminating 3D "flicker."

- Sequential Boot Logic: To prevent PoE network congestion, the Pi utilizes a threaded 10-second staggered boot sequence to initialize both cameras reliably.

## 3.2. The Video Bridge (Multicast Threaded Server)

The Pi runs a multithreaded Python bridge that acts as a high-speed traffic controller.

- Parallel Streams: The bridge hosts two simultaneous raw TCP servers:

  - Port 8085: Dedicated to "The Inspector" feed.

  - Port 8086: Dedicated to "The Site Manager" feed.

- Resource Efficiency: Because of hardware encoding, the Pi CPU usage remains below 10%, providing thermal stability for long-duration operation.

## 3.3. The Communication Layer (Bi-Directional Control)

A dedicated control channel on Port 8088 facilitates the "Human-as-Gatekeeper" protocol.

- Protocol: Raw TCP Socket using UTF-8 String packets.

- Logic: Unity sends a command (e.g., "COMMIT") → Pi captures string → Pi triggers ROS 2 action. This separates control logic from the video stream to prevent packet collision and latency spikes.

## 3.4. The Operator Station (Diegetic Control Room)

The interface is designed as a "Window" into the lunar site rather than a floating HUD.

- Industrial Chassis: The video is projected onto a curved monitor. The virtual distance to the screen will be calibrated 1-to-1 with the physical camera distance to eliminate eye strain.

- Dual-Source Shader Graph: A custom Unity 6 Shader Graph accepts two distinct texture inputs. Using the Eye Index node, it dynamically branches to show the correct camera source to each eye.

- Diegetic UI: View-switching and Robot Actions (Fix/Drop) are handled via World-Space VR Buttons mounted to the monitor's physical bezel.

# 4. Decision Matrix

| Challenge | Discarded Solution | Current Solution (The Choice) |
| --- | --- | --- |
| CPU Bottleneck | Pi-Side cv2.imencode | OAK-D Hardware Encoder: 0% Pi CPU load for video. |
| Visual Flicker | Single-Eye async updates | Sync Node + Double Buffering: Perfect 3D alignment. |
| Data Integrity | Combined Video/Control pipe | Dual Highway (Port 8088): Dedicated Command Channel. |
| OS Compatibility | Raspberry Pi OS (Debian) | Ubuntu 24.04 LTS: Native support for ROS 2 Jazzy. |

# 5. Current Development Phase: Week 2

Milestones Achieved:

- Stable 3D Stereo stream from two physical cameras.

- Successful implementation of the "God Mode" wired backbone.

- Integration of high-fidelity "NASAPunk" control room environment.

- Working VR-to-Pi view switching UI.

- Validated Bi-Directional Command Pipe (Unity-to-Pi trigger).

# 6. Future Roadmap

- Motion Primitives (Week 2): Developing the first movement scripts for the Dobot E6 via the Command Highway.

- Tolerance Engine (Week 3): Implementing Aruco marker tracking on the Pi (utilizing newly freed CPU overhead).

- Digital Overlays (Week 3): Projecting G/Y/R alignment "ghosts" onto the 3D video feed via the Shader Graph.

# Appendix

# How to stream the video.

Phase 1: Physical Power-Up

- PoE Switch: Power On (Wait 20s for cameras to boot).

- HP Dock: Connect to Laptop and PoE Switch.

- Raspberry Pi: Power On.

Phase 2: The Handshake (Laptop PowerShell)

ssh admin@169.254.1.10

Or

ssh admin@Side-Quest1.local

Phase 3: Launching the System
source ~/regolith-env/bin/activate; cd ~/depthai-python; python3 dual_cam_bridge.py

Phase 4: Safety Shutdown

- Stop script: Ctrl + C.

- Safe Halt:

sudo shutdown -h now.

- Wait for the green LED to stop blinking before unplugging.