# Robotic Control & Autonomous Vision build (v3)

Technical Brief

____________________________________________________________________________________

Last edited by: Albert on 17 Feb 2026

Project: Side Quest: The Leaning Tower of Regolith (ARC 2026)

Target Sub-Team: Robotics

____________________________________________________________________________________

# 1. Objective

To develop a modular, scalable robotic control pipeline for a Dobot Magician E6 performing semi-autonomous lunar block stacking under a Hover–Commit supervisory control paradigm.

The system must support:

- Autonomous pick and approach.

- Hover pause for human evaluation.

- Human authorization to Drop or Fix.

- Limited teleoperation (NUDGE mode) during Fix.

- Autonomous continuation to next block.

- Combo-based speed boost mechanics.

- Controlled perceptual uncertainty via drift injection.

The architecture must cleanly separate:

- Vision (pose estimation + drift),

- Control (state machine + sequencing),

- Motion (robot command execution),

- XR (UI and visualization only).

2. System Architecture Overview

2.1 Hardware Stack

- Robot: Dobot Magician E6 (TCP/IP control, Port 29999)

- Control Node: Raspberry Pi 5 (Task Controller + Motion Driver)

- Vision: 2x OAK-D Pro PoE (Inspector + Site Manager)

- XR Interface: Unity (Meta Quest 3 via Link)

2.2 Network Segmentation

- Robot Subnet: 192.168.5.x

- XR / Vision Subnet: 169.254.1.x

- Unity sends high-level intent over TCP (Port 8088)

- Pi executes motion primitives

Important:
Unity does not control raw robot motion.

3. Team Responsibilities

3.1 Vision Sub-Team (The Eyes)

Goal:

Provide reliable 6D block pose estimation relative to the robot base frame.

Responsibilities:

- Stereo processing using OAK-D sensors.

- Aruco-based detection and pose extraction.

- Coordinate transformation into robot base frame.

- Drift injection module (see below).

- Output structured pose data:

  - Pick pose

  - Hover pose

  - Target placement pose

  - Measured placement pose (post-drop evaluation)

Drift Injection Module

Drift is applied:

Detection → Base Transform → Drift Injection → Final Proposed Pose → Control Layer

Drift represents perceptual/world uncertainty, not actuator noise.

Implementation:

- Gaussian offsets in:

  - X

  - Y

  - Yaw

- Z initially excluded (to avoid table collision risk).

- Magnitude parameterized per experimental condition.

Important:
Drift modifies the proposed placement pose only.
Robot motion execution remains deterministic.

3.2 Control Sub-Team (The Brain)

The Pi runs a deterministic Task Controller with a defined state machine.

State Machine

States:

- IDLE

- MOVING_TO_PICK

- GRIPPING

- MOVING_TO_HOVER

- WAITING_FOR_DECISION

- NUDGE_MODE

- PLACING

- RETRACTING

- SAFE_HOME

- FAULT

Core Behavior Loop

- Move to Pick pose (autonomous).

- Close gripper.

- Move to Hover pose.

- Enter WAITING_FOR_DECISION.

At WAITING_FOR_DECISION:

- DROP → PLACING

- FIX → NUDGE_MODE

NUDGE_MODE (Shared Control)

Constraints:

- Discrete button-based.

- XY translation only.

- Optional yaw rotation.

- Z locked.

- Small step increments (e.g., 3mm XY, 2° yaw).

- Precision speed enforced.

Exit from NUDGE_MODE:

- DROP → PLACING

- CANCEL → WAITING_FOR_DECISION

Placement Logic

PLACING:

- Controlled descent.

- Gripper release.

- Retract upward.

After RETRACTING:

If bricks_remaining > 0:

- Automatically proceed to MOVING_TO_PICK.

Else:

- Go to SAFE_HOME.

4. Motion Driver Layer

Responsible for raw Dobot commands:

- ClearError()

- EnableRobot()

- SpeedFactor()

- MovJ / MovL

- RelMovLUser (for nudges)

- Gripper control

- Safe Home pose

This layer must remain deterministic and parameter-driven.

5. Placement Evaluation & Verdict

After each placement:

Vision provides:

- Target pose

- Measured placement pose

The Pi computes:

delta = target_pose - measured_pose

Verdict classification:

- GREEN → within tight threshold

- YELLOW → moderate deviation

- RED → large deviation

Verdict is computed on the Pi.

Unity displays the result but does not compute it.

6. Combo Mode (Speed Boost Mechanic)

If:

- 3 consecutive GREEN placements occur,

Then:

- Activate Combo Mode.

- Increase robot SpeedFactor by +10–20%.

Constraints:

- Boost applies only to travel states.

- Precision states (NUDGE, PLACING) remain at reduced speed.

- Boost resets on YELLOW or RED placement.

Speed boost is system-controlled, not participant-controlled.

7. Safety Architecture

The Dobot E6 provides:

- Built-in collision detection (LED Yellow on collision).

- Alarm state (LED Red).

- Enable state (Green steady).

The Task Controller must:

- Enforce Z-height floor.

- Enforce XY motion envelope.

- Transition to FAULT state on collision or alarm.

- Allow SAFE_HOME routine at any time.

Facilitator may enter workspace only when:

- Robot is Disabled.

- Robot is in SAFE_HOME pose.

Collision detection is treated as a last-layer safety net.

8. System Interfacing (XR)

Unity responsibilities:

- Camera switching.

- Overlay rendering (tectonic zones).

- Intent commands:

  - PICK

  - DROP

  - FIX

  - NUDGE_XY

  - NUDGE_YAW

  - CANCEL

Unity does not:

- Sequence robot motion.

- Compute placement verdict.

- Modify robot speed directly.

9. Development Roadmap (v3)

Phase 1 – Deterministic Motion Primitives

- Implement Safe Home.

- Implement Pick / Hover.

- Implement gripper control.

- Implement Speed profiles (travel + precision).

- Validate NUDGE primitive (RelMovLUser XY only).

Phase 2 – State Machine Integration

- Implement full state transitions.

- Implement NUDGE_MODE.

- Implement DROP & CANCEL logic.

- Implement post-place loop (next brick or safe).

Phase 3 – Vision Pose Integration

- Replace hardcoded poses with vision-derived poses.

- Validate coordinate alignment.

- Integrate drift injection.

Phase 4 – Placement Evaluation + Combo Mode

- Implement Pi-side delta computation.

- Implement GREEN/YELLOW/RED verdict logic.

- Implement streak counter.

- Implement Combo speed boost.

Phase 5 – Robustness & Safety Hardening

- Motion envelope limits.

- Fault recovery procedures.

- Lost connection handling.

- Stress testing repeated cycles.